CPE301 – SPRING 2019

# Design Assignment 2B

Student Name: Brian West

Student #: 5003032874

Student Email: westb2@unlv.nevada.edu

Primary Github address: https://github.com/westbrian2/Spring2019

Directory: Spring2019/DesignAssignments/DA2A_submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components: Xplained Mini board, wires, Logic Analyzer

Block Diagram:

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

None given

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

ASM CODE

.include <m328pdef.inc>

.org 0 ; body of program

jmp main ;

.org 2 ; external interrupt

jmp EX0_ISR

main:

;setting up stack

ldi r20,high(RAMEND)

out sph,r20

ldi r20,low(RAMEND)

out spl, r20

;setting up timer

ldi r20,4 ; prescaler 1024

sts tccr1b,r20;

ldi r20,0x2;INT0 is falling edge triggered

sts EICRA,r20

sbi ddrb,2; portb2 is output

sbi portb,2; turning off led

sbi portd,2; pull up resistor

ldi r20,1<<INT0; enables INT0

out EIMSK,r20

sei; enables interrupts

end:

jmp end

delay:

lds r22,tcnt1h; getting upper bits from counter

lds r23,tcnt1l; getting lower

cpi r23,0x25; comparing against target value

brsh upper

jmp delay ; if target value isn't met restart loop

upper:

cpi r22,0x26

brlt delay

ret

EX0_ISR:

ldi r20,0

sts tcnt1h,r20; resetting timer

sts tcnt1l,r20;

in r21,portb;

ldi r22,(1<<2);

eor r21,r22; toggles LED

out portb,r21;

rcall delay ; starting delay

eor r21,r22

out portb,r22;

reti ; returns

C CODE

#define F_CPU 8000000UL

#include <util/delay.h>

#include <avr/io.h>

#include <avr/interrupt.h>

int main ()

{

DDRB = 1<<2;//PB5 as an output

PORTD = 1<<2;//pull-up activated

PORTB = 1<<2;

EICRA = 0x2;//make INT0 falling edge triggered

EIMSK = (1<<INT0);//enable external interrupt 0

sei ();//enable interrupts

while (1);//wait here

}

ISR (INT0_vect)//ISR for external interrupt 0

{

PORTB ^= (1<<2);//toggle PORTB.2

_delay_ms(1250);

PORTB ^=(1<<2);

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Screenshots from LOGIC

ASM

Time for event is 1.249s

C Code

Time for event is 1.254s

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

The two wires were brought together to trigger the Interrupt.

- VIDEO LINKS OF EACH DEMO

https://youtu.be/3B4we0_-obA

- GITHUB LINK OF THIS DA

https://github.com/westbrian2/Spring2019/tree/master/DesignAssignments/DA2B_submission

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Brian West