CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Brian West

Student #: 5003032874

Student Email: westb2@unlv.edu

Primary Github address: https://github.com/westbrian2/Spring2019

Directory: Spring2019/DesignAssignments/DA3A_submission

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

I did not directly use any pins rather I read the output via the USB connection on the Xplained Mini board.  However, PinD1 was used for transmission.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#define UBRR_9600 103 //Baud rate for 16MHz

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

void USART_init( unsigned int ubrr ); //Sets up usart for use

void USART_tx_string( char *data ); //function that outputs data (usart)

char outint[10]; //for integer

char outfloat[10]; //for float

int num1=66;

float num2=1.75;

int main(void){

sei(); //enable interrupt

USART_init(UBRR_9600); //set up usart

OCR1A = 15625; //setting compare value

TIMSK1=2; //enables match interrupt

TCCR1B=(1<<WGM12)|(1<<CS12)|(1<<CS10); //setting for CTC and prescaler of 1024

snprintf(outint,sizeof(outint),"%d\r\n",num1); //coverts decimal to string to send

snprintf(outfloat,sizeof(outfloat),"%f\r\n",num2); //converts float to string to send

while(1){

}

}

ISR(TIMER1_COMPA_vect){

USART_tx_string("String\r\n");

_delay_ms(50);

USART_tx_string(outint);

_delay_ms(50);

USART_tx_string(outfloat);

_delay_ms(50);

TCNT1=0;

}

void USART_init(unsigned int ubrr){

UBRR0H=(unsigned char)(ubrr>>8); //Setting up

UBRR0L=(unsigned char)(ubrr);

UCSR0B=(1<<TXEN0);//Enabling reciever, transmitter, and rx interrupt

UCSR0C=(1<<UCSZ01)|(1<<UCSZ00); //async  8 n 1

}

void USART_tx_string(char *data){ //sends string

while((*data!= '\0')){

while(!(UCSR0A&(1<<UDRE0)));

UDR0=*data;

data++; //gets next part of data

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Only one Task

- SCHEMATICS

No schematic, Xplained mini was plugged in

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

I did two implementations and this was my favorite, the values are updated after ~1sec of waiting. I did it this way because I am working on a project that needs a similar sampling of input from a load cell. I figured this would be a good way to test the implementation I had in mind.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=xIMbkIHwa8s&feature=youtu.be

- GITHUB LINK OF THIS DA

https://github.com/westbrian2/Spring2019/DesignAssignments/DA3A_submission

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Brian West