CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Brian West

Student #: 5003032874

Student Email: westb2@unlv.nevada.edu

Primary Github address: https://github.com/westbrian2/Spring2019

Directory: Spring2019/DesignAssignments

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Mini Xplained board, power supply, L293DNE, DC motor, switch, potentiometer(5K?)

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void read_adc(void); //gets value from potentiometer

volatile unsigned int adc_temp;

int main(void){

int speed=0;

//set up output/input

DDRD=(1<<6); //setting pinD6 to output

DDRB=(1<<5); //setting pinb5 to output

DDRC=(0<<1); //setting pinc1 as input (PinC0 is ADC)

PORTB=(0<<5); //setting enable pin low

//setting up pin change interrupt

PCICR=(1<<PCIE1); //enabling pins 8-14

PCMSK1=2; //enabling pcint9 (PC1)

//Set up ADC

ADMUX=((0<<REFS1)|(1<<REFS0)|(1<<ADLAR)|(0<<MUX3)|(0<<MUX2)|(0<<MUX1)|(0<<MUX0)); //reference Vcc, left shift(8bits), and PC0 is input

ADCSRA=((1<<ADEN)|(0<<ADSC)|(0<<ADATE)|(0<<ADIF)|(0<<ADIE)|(1<<ADPS2)|(0<<ADPS1)|(1<<ADPS0)); //prescaler=32, enable with no interrupts

//Set up timer0 for PWM

OCR0A=0x88; //set for ~50 duty cycle

TCCR0A|=((1<<COM0A0)|(1<<WGM01)|(1<<WGM00)); //setting to fast PWM, top of 0xFF, and no prescaler

TCCR0B|=((1<<WGM02)|(1<<CS00)); //starts timer

sei();

while(1){

//poll the adc value and update speed based on value

read_adc(); //gets value from pot

if(adc_temp>=0xF2)

OCR0A=0xF2;// max value is 95% of top

else

OCR0A=adc_temp; //assign value

}

}

void read_adc(void) {

ADCSRA |= (1<<ADSC); //start conversion

while(ADCSRA & (1<<ADSC)); //waiting for coversion to finish

adc_temp= ADC;

}

ISR(PCINT1_vect) { //when pinC1 is hit

PORTB^=(1<<5); //toggle enable

_delay_ms(100); //debouncing

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Wave form from midpoint on potentiometer

Quickly turning back and forth

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=YV1AaTyAUWc&feature=youtu.be

- GITHUB LINK OF THIS DA

https://github.com/westbrian2/Spring2019/DesignAssignments/DA4A_submission

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Brian West